Severity Rating Scale

(Should be tailored to meet the needs of your company.)

| Rating | Description | Definition (Severity of Effect) |
| --- | --- | --- |
| 10 | Dangerously high | Failure could injure the customer or an employee. |
| 9 | Extremely high | Failure would create noncompliance with federal regulations. |
| 8 | Very high | Failure renders the unit inoperable or unfit for use. |
| 7 | High | Failure causes a high degree of customer dissatisfaction. |
| 6 | Moderate | Failure results in a subsystem or partial malfunction of the product. |
| 5 | Low | Failure creates enough of a performance loss to cause the customer to complain. |
| 4 | Very Low | Failure can be overcome with modifications to the customer’s process or product, but there is minor performance loss. |
| 3 | Minor | Failure would create a minor nuisance to the customer, but the customer can overcome it without performance loss. |
| 2 | Very Minor | Failure may not be readily apparent to the customer, but would have minor effects on the customer’s process or product. |
| 1 | None | Failure would not be noticeable to the customer and would not affect the customer’s process or product. |

Occurrence Rating Scale

(Should be tailored to meet the needs of your company.)

| Rating | Description | Potential Failure Rate |
| --- | --- | --- |
| 10 | Very High: Failure is almost inevitable. | More than one occurrence per day or a probability of more than three occurrences in 10 events (Cpk < 0.33). |
| 9 | High: Failures occur almost as often as not. | One occurrence every three to four days or a probability of three occurrences in 10 events (Cpk ≈ 0.33). |
| 8 | High: Repeated failures. | One occurrence per week or a probability of 5 occurrences in 100 events (Cpk ≈ 0.67). |
| 7 | High: Failures occur often. | One occurrence every month or one occurrence in 100 events (Cpk ≈ 0.83). |
| 6 | Moderately High: Frequent failures. | One occurrence every three months or three occurrences in 1,000 events (Cpk ≈ 1.00). |
| 5 | Moderate: Occasional failures. | One occurrence every six months to one year or five occurrences in 10,000 events (Cpk ≈ 1.17). |
| 4 | Moderately Low: Infrequent failures. | One occurrence per year or six occurrences in 100,000 events (Cpk ≈ 1.33). |
| 3 | Low: Relatively few failures. | One occurrence every one to three years or six occurrences in ten million events (Cpk ≈ 1.67). |
| 2 | Low: Failures are few and far between. | One occurrence every three to five years or 2 occurrences in one billion events (Cpk ≈ 2.00). |
| 1 | Remote: Failure is unlikely. | One occurrence in greater than five years or less than two occurrences in one billion events (Cpk > 2.00). |

Detection Rating Scale

(Should be tailored to meet the needs of your company.)

| Rating | Description | Definition |
| --- | --- | --- |
| 10 | Absolute Uncertainty | The product is not inspected or the defect caused by failure is not detectable. |
| 9 | Very Remote | Product is sampled, inspected, and released based on Acceptable Quality Level (AQL) sampling plans. |
| 8 | Remote | Product is accepted based on no defectives in a sample. |
| 7 | Very Low | Product is 100% manually inspected in the process. |
| 6 | Low | Product is 100% manually inspected using go/no-go or other mistake-proofing gauges. |
| 5 | Moderate | Some Statistical Process Control (SPC) is used in process and product is final inspected off-line. |
| 4 | Moderately High | SPC is used and there is immediate reaction to out-of-control conditions. |
| 3 | High | An effective SPC program is in place with process capabilities (Cpk) greater than 1.33. |
| 2 | Very High | All product is 100% automatically inspected. |
| 1 | Almost Certain | The defect is obvious or there is 100% automatic inspection with regular calibration and preventive maintenance of the inspection equipment. |